PURGENVENT Model 7940BLE

- 2.1	SPRINKLER SPECIALTY PIPE FITTINGS

Main Line Air Vent:

Product: M7930BLE Branch Line Automatic Air Venting Valve. The separation chamber and air scoop is designed to force a separation of air from the water as the system fills.

Standard: UL's "Fire Protection Equipment Directory" listing or "Approval Guide," published by FM Global, listing.  NFPA 13, 13R, 13D (as applicable).

Includes M7900AAV.

Pressure Rating: 300 psig (2070 kPa).

Branch Line Extension: carbon steel body with threaded fitting for machined brass Air Scoop attachment. Powder coated Safety Red.

** NOTE TO SPECIFIER ** Delete inlet and outlet size not required.

Inlet and Outlet Size:

2 inches (51 mm) Grooved ends.

2.5 inches (64 mm) Grooved ends.

3 inches (76 mm) Grooved ends.

4 inches (102 mm) Grooved ends.

6 inches (152 mm) Grooved ends.

8 inches (203 mm) Grooved ends

- inches (203 mm) Grooved ends. ** NOTE TO SPECIFIER ** Delete product not required.

Optional Water Collection Accessories:

Product: Model 7920AI. Water Collection Assembly with Audible Alarm.

Product: Model 7925VI.	Water Collection Assembly with Visual Indicator.