Model 5300 COLLECTanDRAIN

- 2.1	SPRINKLER SPECIALTY PIPE FITTINGS

Auxiliary Drain and Condensation Collecting Assemblies for Dry Pipe and Preaction Sprinkler Systems:

Manufacturers:  Subject to compliance with requirements, available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

AGF Manufacturing Inc.; Model 5300A Galvanized Preassembled COLLECTanDRAIN™.

AGF Manufacturing Inc.; Model 5300B Black Steel Preassembled COLLECTanDRAIN™.

Standards:

UL's "Fire Protection Equipment Directory" listing or "Approval Guide," published by FM Global, listing.

NFPA 13 and NFPA 25.

Components:

Two 1-inch MIPT x FIPT Brass quarter turn ball valves with chrome plated balls.

Two 1-inch by 2-inch Galvanized Bell Reducers.

2-inch by 12-inch Condensate Nipple.

1-inch Galvanized Square Head Plug.

Vinyl Identification Label